### Concordia University

Life Cycle Analysis of Aluminum Can

ENGR 202 – Summer 2013

Assignment #2 Submitted to Professor John Hadjinicolaou Sustainable Development and Environmental Stewardship Section: AA

Due date: June 12, 2013

STUDENT NAME: STUDENT ID

Muneeb Manzur: 6775985

Ahmad Ali Khan: 6772641

Danny Thomas: 1310992

Ravikumar Patel: 6645208

Abhishek Appadoo: 6540635

Rajithan Sivapatharajah: 6592031

Ahmed Abdalla: 9619097

Abdul Malik 5813492

Detailed Tasks Separation:

Muneeb Manzur

- Team Leader

- Produced inventory, impact, and improvement analysis of inner Coating

- Process, Energy use, Wastes.

- Final Editor; Layout.

Ahmad Ali Khan

- Assistant.

- Manufacturing Process; Time for each step.

- Moulding of Aluminum Can.

- Generated Solid Works Life Cycle Assessment report of aluminum can.

Danny Thomas

- Aluminum Preparation process for producing Aluminum sheets.

- End of Life of Aluminum Can and Decomposition time, Hazardous Effects on Human Health and Ecosystem.

- Comparison between stages of packaging of beverages.

Ravikumar Patel

- Assistant

- Communicator: Facebook Group; Communication and meeting dates.

- Shipping, Transportation and Packaging.

- References and in text citation.

Abishek Appadoo

- Assistant.

- Abstract, Introduction and Conclusion.

- Disposal, Recycling Cleaning Process; Waste Product and Energy.

- Difficulty of Recycling Aluminum.

Rajithan Sivapatharajah

- Refining Aluminum.

- Reusing Aluminum.

- Raw Material, Mining of Aluminum + Characteristic of Aluminum

- Waste, Energy Use, Extraction process.

Ahmed Abdalla

- Assistant.

- Lid and Cap, Forming Process; How is it attached to base of can + Energy use.

- Material of Lid and Cap; Comparison with material of can.

- Filling of a can; assembly of lid and cap.

Abdul Malik

- Editor and Supervisor.

- Outline and Role Assignment

- Final Design; Exterior design/colors.

- Material used and painting process.

## Abstract

The life cycle of aluminum cans affects the environment every step of the way. During the cycle, elements are being consumed and emissions are released into the atmosphere while using fuel to achieve the process. All of this together leads to the result of a global effect. It starts with mining the aluminum ore known as bauxite and then has to undergo certain processes to convert it to aluminum ingot blocks which is an industry standard. These blocks are then rolled into thin sheets which are used for shaping and producing the cans through several stages. When it’s ready, they’re filled with a beverage to be sold and consumed. When the beverage is consumed, most of the cans then start the recycling process. This eventually makes it back to new aluminum ingots which repeat the cycle. Traveling and packaging are present between each stage with major plants and facilities spread around the globe.

Figure 1: Aluminum Can Lifecycle [10]

TABLE OF CONTENTS

1.0.0	INTRODUCTION (Abishek Appadoo)	8

2.0.0 PREPARATION AND DESCRIPTION OF THE ALUMINUM CAN (Rajithan Sivapatharajah)	9

2.2.0 Refining Process	10

2.2.1 Waste Management Strategies	12

2.3.0 Aluminum Preparation for Rolling	12

2.4.0 Environmental Impact on Aluminum Sheet Production	15

2.5.0 Characteristics and description of Aluminum	17

3.0.0 MANUFACTURING PROCESS OF THE ALUMINUM CAN (Ahmad Ali Khan)	19

3.1.0 Main Can Body	19

3.1.1 LCA - Aluminum Trimmings	21

3.1.2 – LCA – Lubricant	22

3.1.3 – Energy Consumption	22

3.2.0 Inner Coating (Muneeb Manzur)	24

3.2.1 Inventory Analysis	24

3.2.2 Impact Analysis	25

3.2.3 Health Impact	25

3.2.4 Environmental impact	27

3.2.5 Evident Impacts	27

3.2.6 Improvement Analysis	28

3.3.0 Aluminum Lid (Ahmed Abdalla)	31

3.3.1 Material Selection	31

3.3.2Manufacturing Procedures	31

3.3.3 Environmental Impact	33

3.3.4 Improvements and Alterations	34

3.4.0 External Can Design (Ravikumar Patel)	35

3.4.1 Conventional Methods	35

3.4.2 Economic and Energy Comparisons	36

3.5.0 Transportation	37

3.5.1 Reduction of Transportation Emission	37

4.0.0 END LIFE OF ALUMINUM CAN (Danny Thomas)	39

4.1.0 Decomposition Time	39

4.2.0 Comparison between stages	39

4.2.1 Glass bottles	40

4.2.2 Plastic Bottles	41

4.2.3 Aluminum Cans	42

4.3.0 Disposal (Abdul Malik)	45

4.3.1 Waste	46

4.3.2 Public Recycling Program	46

4.3.3 Treatment Plants	48

5.0.0 CLOSING REMARKS (Abishek Appadoo)	52

6.0.0 ENVIRONMENTAL BENEFITS	53

REFERENCES	54

LIST OF FIGURES

Figure 1: Aluminum Can Lifecycle	3

Figure 2 - Bauxite Ore	9

Figure 3: The Distribution of Bauxite ore Reserve	10

Figure 4: Aluminum Sheet Flow Chart	18

Figure 5: Ironing Process from Blanks to Final Shape	19

Figure 6: Top Edge Trimming and Forming	20

Figure 7: Can Coating	25

Figure 8: Chart Showing NTP Conclusion on BPA	26

Figure 9: Illnesses Associated With BPA, and Their Occurrence Probability	26

Figure 10: 3D Chemical Structure of biphenyl A.	27

Figure 11: Spatial distributions of the mean concentrations of bisphenol	28

Figure 12: Summary diagram of the Improvement Analysis steps	30

Figure 13: Printing Automation	35

Figure 14: Glass Bottles	40

Figure 15: Plastic Bottles	41

Figure 16: Aluminum Cans	42

Figure 17: Pressed Cans	44

Figure 18: Treatment Process 	48

Figure 19: Aluminum Can Lifecycle Process 	52

LIST OF TABLES

Table 1: Electrical Power Consumption	15

Table 2: Parameters for Production	15

Table 3: Emissions and Outputs during Electrolysis	16

Table 4: The comparison material weight	17

Table 5: Summary of quality assurance checks on in-coming containers at food packers	32

Table 6 - Factors accelerating external corrosion	33

Table 7: Economic Comparison	36

Table 9: U.S. Aluminum Beverage Can Recycling	45

Table 10: Energy used to prepare aluminum scrap per 1000kg	49

Table 11: Emissions, from processes, into the air per 1000kg	49

Table 12: Energy used to cast 1000kg of aluminum rolling ingot	50

Table 13: Top 8 pollutants emitted to the air per 1000kg of aluminum rolling ingot	50

Table 14: Top 8 pollutants emitted into water per 1000kg of aluminum rolling ingot	51

# INTRODUCTION

The life cycle assessment (LCA) is used to scientifically determine the environmental impact of a produced object from its extraction to decomposition stage. The environment and the global ecosystem are all interconnected with itself and even the universe. The LCA takes into account many of the direct and indirect relations every stage a specific product undergoes. These results are used to accurately determine how much of an influence it has on the global environment and how it should be used and monitored. It starts by extracting aluminum ore which is primarily obtained from bauxite through mining operations.  The aluminum is then processed into large industry standard bricks called ingots to be used for a wide variety of commercial products. For aluminum cans to be made, the ingots are rolled into thin sheets where multiple disks are punched out which later forms the can. Eventually, these disks are converted into full cans which are filled with the desired beverage for consumers around the world to enjoy. When the cans are disposed, there are strategies to collect these cans since they are completely recyclable. Also when aluminum cans are collected, cleaned, melted down, and casted into aluminum ingots; it ends up saving on mining and energy costs. Almost every step along the way, the aluminum is packaged and shipped which adds to the materials used, emissions expelled, and energy consumed.  This assessment will explore both direct and indirect impacts on the environment due to the production and use of aluminum beverage cans and their alternatives.

# 2.0.0PREPARATION and DESCRIPTION OF THE ALUMINUM CAN

2.1.0 Primary Raw Material for Manufacturing

A can is described as a metallic container used to preserve air-sensitive solids and liquids such as paint, glue, beverages, or food for a certain period of time. In most cases, the main reason for using cans is to retain or stock objects for a long period of time by vacuum packaging.

Several steps and processes are implemented throughout the life cycle of the can. First is to check what kinds of raw materials are used by other companies and compare. The main material of a can is generally aluminum which is the third most abundant element in Earth`s crust (about 8.05%). It’s also the second most widely used material in the world. Aluminum industries started developing rapidly in the late 19th century. Approximately 10-50% of aluminum has been used for containers especially cans. [1]

Figure 2 - Bauxite Ore [1]

Aluminum is extracted from bauxite ore which consists of 40~60 % aluminum, 1~15 % silica, 7~30 % iron oxide, and 12~30 % combined H2O. In addition, it’s also comprised of various minerals such as gibbsite, boehmite, diaspore, goethite, and hematite. Bauxite ore is extracted from earth`s crust 600 meters below ground level. The average depth for its extraction deposit is about 80 meters. Mostly, it is taken out in open-pit mines that are formulated by removing and excavating the burden located on the surface of earth. After eliminating the surface, explosives and grinding processes are used to form paths through which mining, extracting and dust controlling teams can actually approach bauxite ore deposit. Dust controlling teams are technicians that limit the emission of dust from grinding and drilling into the crust.

The extracted bauxite ore goes to a factory which is normally few meters away from the mine. It’s where ore undergoes ‘beneficiation’ that is used to remove the impurities from the bauxite ore. This is a prerequisite procedure for the refinery of the ore. After this procedure, beneficiated bauxite ore is shipped to other factories to be refined as aluminum. [2]

Across continents (%)

In order to extract one ton of aluminum, four to nine tons of bauxite ore are needed. Across the world, the total reserve of bauxite ore is approximately estimated to be 28,000,000 tons which is only enough for 120 years. The reserves are mostly located in Africa, Oceania and mainly four countries which are Australia, Brazil, Guinea, and Jamaica. The rest of the reserves are deposited in 27 other countries. [3]

## 2.2.0 Refining Process

Figure 3: The Distribution of Bauxite ore Reserve. [1]

Across countries (%)

Aluminum is obtained in two steps from bauxite ore. The first step is a purification of the ore by removing impurities such as ferrous oxide, silica and titanium dioxide. The Bayer process is the industrial norm to isolate aluminum oxide and consists of four main processes: digestion, filtration, precipitation and calcination. Bauxite ore is positioned and mixed with sodium hydroxide in a wet mill to form slurry. The digestion process is the

heating and pressurization of this mixture in large tanks

to facilitate the reaction:

AlOH3s+ALOOHs+2NaOH aq.+H2O→2NaAlOH4(aq.)

Sodium hydroxide and sodium oxy hydroxides are dissolved and the alkaline saturated solution of sodium aluminates is diluted. Impurities of ferrous oxide and titanium dioxide are suspended. The slurry is then pumped to filtration systems such as sand traps, thickeners and finally cloth filters to trap impurities of different masses. The separated solution is cooled and sent to precipitators whereas all the solid sediments called the Red Mud is then prepared for disposal. [4]

Aluminum hydroxide is added to the solution to initiate the crystallization of aluminum hydroxide from the solution according to the reaction:

NaAlOH4aq.→AlOH3s+NaOH(aq.)

If crystalline agglomerates are large, they are easily filtered out. Instead of being disposed, the unusable finer alumina is used to initiate the precipitation of the next batch. This makes the Bayer process resource efficient. Washed aluminum hydroxide is calcined in rotary kilns or stationary fluidized-bed flash calciners that break down the hydroxide form to the oxide and water vapour form. [5]

AlOH3(s)→Al2O3(s)+H2O (g)

The process to obtain aluminum metal from alumina from the Bayer process is electrolysis by the Hall-Heroult process. A molten solution of cryolite Na3AlF6 and alumina is electrolysed in a carbon lined vat which constitutes the cathode. AlF3 is added to this mixture to lower the melting point of cryolite. An array of carbon bars are immersed in this electrolyte and will be the anode. When electricity is passed inside the mixture, the O2- ions move towards the graphite bars and react as:

C+2O2-+4e-→CO2

Al3+ ions are separated from alumina and move towards the lining of the vat where it gets neutralized to liquid aluminum. The liquid aluminum is denser than cryolite and settles at the bottom. It is siphoned out and directed away to where they are cooled and formed into aluminum ingots.

### 2.2.1 Waste Management Strategies

Depending on the nature of bauxite, only a maximum of 50 parts by weight of alumina are produced from 100 parts of bauxite by weight. [6]This alumina is relatively pure and can be smelted to obtain pure aluminum. Evidently, 50 or more parts of impurities are a by-product of the Bayer process. The thick slurry called “Red Mud” presents a waste management concern due to its high rate of accumulation since approximately 30 metric tonnes/years is produced globally. [7] At present, it is disposed in landfills. Due to high basic properties (pH 10-13), red mud landfills are not arable and are uninhabitable. A small fraction of it is recycled for road construction or as an additive for the cement industry. [8]

Due to anode reactions in the Hall-Heroult process, exhaust gases and fumes are produced. The chief gases are CO2, condensed hydrocarbons, fluorocarbons and hydrogen fluoride. [9] Since HF gas is highly corrosive, correct disposal requires neutralization of the HF to sodium fluoride. Dry scrubbing is another method where alumina is used to absorb HF making AlF3 which is reused in electrolysis. [9] The particulates of solid fluoride and alumina are also captured and reused using electrostatic or bag filters. Fluoride emissions are reduced by immersing green poles in the electrolyte and liquid aluminum. [9] Prebaked anodes reduce the production of hydrocarbons because they combust to form CO2 and water vapour. [9]

## 2.3.0 Aluminum Preparation for Rolling

After being extracted for the mine, the aluminum comes in the form of aluminum oxide/alumina which is melted and purified. This is done by sending an electrical current through the solution while in its liquid state which is the electrolysis. [2]An alloy is then produced to get better properties. This helps in the manufacturing process and the added components are 5.5% of magnesium, 1.6% of manganese, and 0.15% of chromium. According to MSDS from aluminum producer Alcoa, the alloy can contain some trace amounts of iron, silicon and copper. [2]

Then the molten alloy is then fluxed which is the extraction of dissolved hydrogen which emulsifies into molten aluminum. When it solidifies the hydrogen, it will form bubbles or pores into the solid aluminum. Hence to remove the hydrogen, dry hydrogen-free gas is passed through the molten gas. Then the hydrogen diffuses into this gas and is carried away. An inert gas such as nitrogen is used for this process.

Afterwards there is the filtration step to remove all other impurities by using deep bed filters which typically consist of layers of media such as high density alumina flakes or spheres. A sample analysis is then preceded, if its composition is not correct; it is corrected and retested. Now that we have the correct alloy it is ready to be made into sheets for the can manufacturing. There is two ways to make the required sheet:

- Continuous  casting of sheet or plate

- DC ingot casting

Continuous casting takes the molten alloy and solidifies it in a continuous strip. There are 6 different techniques but the common feature is that the sheet is produced directly from molten metal and coiled. Although actual thicknesses vary by the method and producer, “as-cast” continuous cast strip is less than 1 inch (25 mm) thick, compared to the 30+ inch (760+ mm) thick ingot produced via the DC ingot process. [2] It’s different from DC ingot casting because it’s thicker and thus, the cooling of the sheet is produced faster whereas DC ingot casting is more complex. The three significant differences between continuous casting and the more traditional DC ingot casting are:

- Significantly reduced hot working in continuous casting compared to DC ingot casting.

- No homogenization between casting and hot rolling.

- No scalping between casting and hot rolling. [11]

The two main steps that characterise DC ingot casting is the scalping and preheating process. The scalping process is the removal of irregularities and some impurities on the surface of the ingot for the press rolling process. And afterwards, there is the preheating process which has several important purposes such as:

- Raising the alloy’s temperature above the recrystallization temperature to prevent cold working (strain-hardening) from taking place during rolling

- Homogenization of the overall alloy.

- Softening the ingot and thereby, minimizing the mechanical force necessary to reduce its thickness by rolling.

4. Relieving stress in the ingot.

Most aluminum used in processes use DC ingot casting because of a perfect result. However, continuous casting demands less energy and work because it costs less for making simple things such as cans. [11]

DC ingot casting of aluminum cans

## 2.4.0 Environmental Impact on Aluminum Sheet Production

Aluminum is a primary material use in many field and humans everyday life. Hence aluminum sheet production is done at a great scale thus every step must be carefully surveyed and with many factors to consider.

First of all to melt the aluminum, purify it and make it go through the steps required and work the tools demand a certain amount of energy which can be obtain with through an electrical power source such as hydroelectric generation which unfortunately no accessible everywhere and hence have to be generated oil fuel will be needed or coal and also for the smelting and working the machines and tools a lot of heat and energy is required so natural gas or again oil fuel or coal could be used which overall will create pollutant that must be managed.

PE Americas give us a breakdown of electrical power (MJ) consumed in electrolysis by energy source in the world just for the electrolysis step.

| Energy source | Hydro | Coal | Oil | Natural Gas | Nuclear | Total |
| --- | --- | --- | --- | --- | --- | --- |
| - | 194187 | 95334 | 2327 | 29997 | 17121 | 9 338966 |

Table 1: Electrical Power Consumption [2]

This table shows the parameters for the production of 1000 kg of primary aluminum ingot in North America.

| Inventory parameter | Alumina Refining | Electrolysis | Cast house | Total |
| --- | --- | --- | --- | --- |
| Primary Energy Demand(GJ/ton) | 27.36 | 124.27 | 2.36 | 153.98 |
| Non-renewable (GJ/ton) | 27.08 | 74.52 | 2.12 | 103.73 |
| Renewable(GJ/ton) | 0.28 | 49.75 | 0.23 | 50.25 |
| CO2 emissions (ton CO2/ton) | 2.03 | 7.47 | 0.14 | 9.63 |

Table 2: Parameters for Production [2]

With all this work done at such temperatures and producing gas emission such as carbon dioxide that can lead to monoxide; an air pollution control system is required.  Plus all the extracted impurities and the water used for cooling system and other purposes also have to be controlled.

This table show all the emission and outputs produced in the electrolysis process (in kg):

| Emissions to air | Amount |
| --- | --- |
| Carbon dioxide | 1181.6 |
| Fluorides | 0.589 |
| Benzo{a}pyrene | 0.002 |
| Polycyclic aromatic hydrocarbons | 0.216 |
| Tetrafluoromethane | 0.112 |
| Hexafluoroethane | 0.0112 |
| Emissions to water |  |
| Fluoride | 0.037 |
| Oil (unspecified) | 0.005 |
| Solids (suspended) | 0.067 |
| Hazardous waste |  |
| Carbon (unspecified) | 5.45 |
| Sludge | 12.83 |
| Water [Water] | 11860 |

Table 3: Emissions and Outputs during Electrolysis [2]

The manufacturer of the tools will also demand a lot of resources and technologies for these tools have to last long for a heavy load of work.

Which bring us to an important part which is the maintenance, indeed all these process use a lot of lubricant (oil and water) and other chemical that are not only used for lubrication but also for the aluminum finish and protection.[11]

## 2.5.0 Characteristics and Description of Aluminum

Aluminum is an ideal element for cans due to its corrosion resistance, lightness, high heat conductivity, and elasticity. Most cans are used to preserve freshness or original states of the product that are vulnerable to exposure of air and light for a short or a long period of time. To retain items for a long time, the container must be resistant to chemical reactions which can happen due to the contact between materials inside the container, and objects inside; especially corrosion which releases toxic substances. Aluminum has thin natural protection film on its surface which protects the product from external corrosion upon contact.

Also, aluminum is one of the lightest elements. This characteristic also encouraged companies to use it to produce containers such as cans. Compared to other materials, Aluminum is the most favourable for can manufacturers to create efficient containers.

The table below proves this statement by comparing Aluminum to other material.

|  | Specific gravity |  | Approximate weight Per cubic foot (lbs) |  | Volume per 1,000 Pounds (cu. ft.) |
| --- | --- | --- | --- | --- | --- |
| Aluminum | 2.71 |  | 170 |  | 5,88 |
| Steel | 7.85 |  | 490 |  | 2.04 |
| Copper | 8.94 |  | 560 |  | 1.78 |
| Zinc | 7.14 |  | 445 |  | 2.24 |
| Lead | 11.34 |  | 700 |  | 1.43 |

Table 4: The comparison material weight

This proves that aluminum is relatively lighter and more abundant than other material; which makes it the most efficient material to use for manufacturing containers. In addition to this, the elasticity of aluminum is very favourable. It can be manufactured in any shapes and forms because of its thermal conductivity; which is 0.51~0.53. Iron`s thermal conductivity, for example, is 0.113 which is relatively higher than other materials. This makes it unfavourable to use. Moreover, aluminum is non-magnet which adds more flexibility to its manufacturing and storage processes. To sum up, aluminum, which is extracted from bauxite ore, is an ideal raw material for cans due to its characteristics; corrosion resistance, lightness, heat conductivity and elasticity. It plays an efficient role as containers since it preserves the product for a long period of time.

Aluminum Oxide/Alumina

Oxygen + Impurities

Recycled aluminum

(50%of overall processed aluminum& 95% less energy to process)

5.5% Magnesium

1.6% Manganese

0.5%chromium

Smelting and electrolysis

Impurities

Fluxing and filtration

ORR

-Reduced Hot working

-No need of homogenization

-No scalping

Better quality of alloy but more step/energy needed

Continuous Casting of sheet or plate

DC Ingot Casting

Scalping

Preheating

Aluminum sheet

Press Rolling

Figure 4: Aluminum Sheet Flow Chart

# 3.0.0 MANUFACTURING PROCESS OF THE ALUMINUM CAN

## 3.1.0 Main Can Body

The manufacturing of the main body of the can is simple process that takes seconds to manufacture. It is a highly material efficient method that does not generate much waste product. But like every other method of fabrication, there is a certain amount of waste in the form of energy, leaks, and evaporation. [12]

The roll of aluminum is fed into rollers that will unroll the thin sheet of aluminum. At this stage, the product is lubricated with synthetic lubricant. The first step of forming the can is punching blanks. Dies press down on the sheet aluminum and cut out round disks. These blanks are what will be formed into the well-known shape of a soda can. [12] The scrap aluminum is collected for recycling. The discs are then further pressed into small cups. This procedure is called drawing. This is actually done at the same time as the disks are punched out of the sheet. This reduces the need of an extra machine, and reduces the amount of energy required to do both processes.

The cups are then moved along to another press. This press forms the shape of the can as we know it through a process called ironing. The cup is fitted into a can shaped negative. A die then pressed the cup into the negative with a large amount of force. Through this process, the sides of the cup get stretched until they are very thin. This is done is one fifth of a second. A round headed die then punches the bottom of the can into a dome to give the thick bottom a better geometrical strength to support the liquid that will fill it. [12] The process is depicted in the image below:

Figure 5: Ironing Process from Blanks to Final Shape [38]

The whole ironing process requires very generous amounts of lubricant to be sprayed all over the dies and can. If the lubricant was not used, this process would not be possible for a few reasons. First of all, there would be too much friction between the dies and aluminum. This would mean that the dies wouldn't be able to slide on the can walls while forming and would tear the material. Furthermore, the friction would cause a hike in temperature which would cause abnormalities and uneven properties in the aluminum. The lubricant, a mix of water and synthetic oil, is a critical component to the soda can fabrication.

Once out of the ironing press, because of the strong shear forces on the can, the top rim is left waxy and rippled. This is obviously not desired so the top is trimmed off and the excess aluminum is collected for recycling.

Figure 6: Top Edge Trimming and Forming [38]

The cans then move on to a washer and get washed with Hydrofluoric acid and water to get rid of all the excess lubricant. After getting their outride painted and their inside coated with varnish (these steps will be discussed further on,) the cans pass through a necker that slowly bends the top of the cans inward. Finally, they pass in another machine that creates a flange on the rim of the can to facilitate the attachment of the cover once the can is filled. That completes the manufacturing of the main body of the soda can. [12]

### 3.1.1 LCA - Aluminum Trimmings

For life cycle analysis (LCA purposes, the elements of this process to be taken into consideration are aluminum trimming recycling, lubricant use and removal, and energy use.

Aluminum trimmings are created at two stages of the can production: when the blanks disks are punched out of the sheet and when the top rim of the can is cut off after ironing. Aluminum is a highly recyclable material so this scrap aluminum gets shipped back to the smelting plant to get smelted and reformed into brand new rolls of sheet aluminum. Typically these two facilities are quite close to each other to limit transportation fees. This is good news for the environment because fuel consumption and greenhouse gas emotions are kept to a minimum.  In Quebec, there is a clear example of this plant proximity. PPG Chlor-Alkali & Derivatives is an industry leader in the soda can fabrication industry. They have a plant situated at Beaharnois Quebec. Beauharnois is also the location of a big Alcoa Aluminum production location. The two plants are literally a few blocks from each other. For this reason we will neglect the transportation of the scrap martial to the smelting plant. Aluminum smelting itself thought cannot be ignored. This process is quite simple; the aluminum gets heated until it is liquid and then reshaped into thin sheets. Aluminum smelting is a very polluting process and can be hazard to plants and wild life if not contained properly.[14] Pollutants released by this process are fluorides (hydrogen fluoride), sulphur dioxide, carbon dioxide, volatile organic compounds, and alumina. Both greenhouse gases and toxic chemicals are found in these emissions. Plants that are well maintained and controlled can emit as little as 0.5 kg of pollutants per ton of aluminum whereas plants that operate under slacker regulations can emit up to 4.4kg per ton. Energy is also required for the smelting; about 15.2 to 15.7MWh of energy per ton. [14] This is enormous. Reducing the amounts of aluminum trimmings is primordial for reducing the environmental impact of the can.

### 3.1.2 – LCA – Lubricant

The second LCA element is the lubricant. Lubricant is also applied at two steps in the process: [it is layered evenly on the sheet of aluminum when being unrolled and then more is used during the ironing process. Industrial lubricants come in two types: petroleum base and water base. Petroleum based lubricants are the traditional choice but are quite harmful for the environment. These days though, the industry opts mostly for mater based lubes. These substances dissolve in water thus can relatively easily be contained within a process. In the soda can manufacturing, the lubricant is collected and reused, but some stays adhered to the can. This excess must be washed off through a hydrofluoric acid spray wash and then a water rinse. This process generates waste HF, lube and contaminated water. [2] The hydrofluoric acid eats away a very thin layer of the aluminum which mixes with the washing fluid wastes. These products cannot be reused and are either ditched as runoff water or sent to the water treatment plant through aqueducts. Numerical information about the runoff is scarce. Other chemicals that are part of the lubricant get washed away in this water such as fluorides and sulphites. Nevertheless, this pollution is present and bears an impact on the environment. [2]

### 3.1.3 – Energy Consumption

Lastly is the energy consumption. Forming the can is not a very energy intensive process compared to the whole aluminum production from the ore itself. As mentioned previously, soda can factories are usually located in the vicinity of aluminum smelting plants. Since smelting is such an energy thirsty industry, they require a source of reliable, infinite, and high power energy. For this reason they are almost always located next to hydroelectric dams. The can factory uses electricity from these dams to fabricate their products. No emissions are caused by the energy used. Aquatic life are the only one impacted by need of energy by having their streams blocked off and turned into lakes that flood hundreds of square kilometers of beautiful liveable land depending on location. Using recycled aluminum as opposed to raw ore to fabricate cans reduce the energy consumption of the fabrication process by ninety five percent. This is enormous. The impact that the energy requirement has on the environment would be almost negligible if all aluminum was created from recycled aluminum. A circular material cycle would make a vast difference on the environment. [2]

To wrap up, the Life Cycle Analysis of the production of the soda can's main body is impacted by three aspects of manufacturing. The smelting of the aluminum trimmings that are not used impact the environment mostly by air pollution releasing sometimes dangerous amounts of toxic products that can kill fauna, flora and cause serious harm to human wellbeing. The use of lubricants creates runoffs that contaminate water with substances such as aluminum, hydrofluoric acid, fluoride, and sulphites. Modern and well regulated facilities minimize this waste though and reuse a great amount of their lubricant; thus, sparing the environment of this waste. And the most obvious: energy consumption. The type of energy used most commonly in the industry is hydroelectric which has only a small impact on landscape and aquatic life.

The fabrication of the soda can's body is responsible for a very minimal portion of the life cycle analysis of the product compared to material creation and disposal.

## 3.2.0 Inner Coating

### 3.2.1 Inventory Analysis

Interior coating, also known as inner coating, comestible polymer coating and can lining, constitutes of a range of materials used to shield the interior of aluminum cans. The primary aim of this operation is to protect the can from dissolving into the acidic content of the container. As a result, the container’s chemical composition, taste, smell and freshness will be assured to remain in an optimum condition

In the past, organic resins were derived from natural substances extracted from certain trees such as pine trees. Those Ingredients were used to coat the interior of a container by following certain spraying techniques. However, currently, the majority of coating substances in use are polymers derived from Petroleum products. Throughout the history of beverages industry, companies have been modifying the composition of the coating materials in order to adapt to the safety regulations. In the present, what is known as cross-linking reactions are used to create interior coatings from epichorohydrin-Bisphenol A cross-linked with urea-formaldehyde or vinyl-resins.

The Bisphenol A (BPA) resins, which are called epoxy resins, represent the majority of coating materials used in the interior layering of aluminum cans. The quantity or the thickness of BPA used in the industry remains undetermined. In fact, independent consumer protection companies have found traces of BPA in a sample of cans to be five times higher than the average residue found in similar rival brands.

Spraying Steps

- The aluminum can open side is advanced towards the rotating spray shaft.

- The epoxy resin is ejected in three different directions in order to fully coat the container.

- The cans are then cured thermally and left to dry, so that the epoxy resin will bind with the interior surface.

Interior coatings are applied to cover the interior of the container by rotating either the can or the spraying head.

Spray shaft

Angular Motion

Translational motion

Figure 7: Can Coating [18]

### 3.2.2 Impact Analysis

The interior coating application of BPA in the aluminum cans industry in general, raises questions in regards to the safety of utilizing BPA for internal coating. Those questions conclude that the BPA can have damaging impact on both our health and welfare. The points below summarize the Health and the environmental impact of BPA on our lives.

### 3.2.3 Health Impact

It is discovered that BPA has the potency to contaminate the content of many beverages cans. Therefore, the chance of developing genetic disorders and cancerous cells in the body could rise. Additionally, it can also increase the probability of immature birth cases as well as mental diseases. It is classified as a low risk contaminant. Nevertheless, many consumers believe that extra attentions are needed to make the phlegmatic regulations associated with the use of BPA more stringent. Below is a diagram describing the potential health risks associated with Bisphenol A (PBA)

Figure 8: Chart Showing NTP Conclusion on BPA. [14]

Underneath is a graph indicating statistical data, which shows the chances of developing different types of diseases linked to BPA exposure. [14]

Figure 9: Illnesses Associated With BPA, and Their Occurrence Probability [17]

### 3.2.4 Environmental impact

The IUPAC molecular name of Bisphenol A is 4, 4’-(propane-2, 2-diyl) diphenol, and its chemical formula is (CH3)2C (C6H4OH) 2.Thefollowing 3D chemical structure is a visual representation of a bisphenol A molecule.

Figure 10:3D Chemical Structure of biphenyl A. [15]

In the Unites States alone, the production of Bisphenol A exceeds two billion tons a year. This fact is devastating, because epoxy resins are hard to recycle, and has no further use after their initial application.

### 3.2.5 Evident Impacts

The major effect exists because the recycling process of aluminum cans involves the removal of the epoxy resins by thermal elimination. This incineration processes emits to the atmosphere carbon dioxide along with other by-products of the uncompleted combustion.

The Secondary environmental impact is the escapement of BPA in the epoxy resins from the surfaces of cans that have been thrown away, and similarly, from beverages’ manufacturing facilities. Since BPA is categorized under the group of aerosols, their accumulation in the atmosphere is frustrating and adds to the problem of global warming.

Figure 11:“Spatial distributions of the mean concentrations of bisphenol

A (BPA) in atmospheric aerosols collected at different locations in the world” [14]

The Third impact is both economic and environmental at the same time. The incineration process of recycling aluminum cans requires considerable amount of energy. It’s a costly operation, and it’s surely the reason preventing the recyclability of cans in cities with limited supply of electricity. Therefore, it’s not surprising that many cities simply discard their cans in dumb landscape instead of recycling them.

### 3.2.6 Improvement Analysis

The window of improvement in the area of interior coating is very narrow. Perhaps applying less layers of Bisphenol A (BPA) coating might reduce its environmental and health impact. Likewise, adopting an energy efficient recycling technique rather than the incineration would possibly make the whole process much more cost efficient. [14]

Currently, the major three recycling practices implemented by the industry are; Incineration, Pyrolysis and Fluidized Bed using Chloride.  So far, neither one of them is energy efficient nor environmentally friendly. [14] The timing of a public anxiety regarding Bisphenol A (BPA) health risk couldn’t be better. In fact it has helped to proliferate the research done in Biodegradable Lining Materials; a technology if been implemented, will eliminate the scrapping stage of the recycling process of aluminum cans. [17]

Many health experts believe that by substituting BPA epoxy lining with another BPA-free alternative, we will surely be able to eradicate the harmful effects of Bisphenol A from our beverages. Unfortunately, up until this date, there are no any feasible alternative that is as durable and cost efficient as BPA epoxy resins. Even thought, there are several safe to use substitutions that can replace the BPA coating, they all tremendously increase the production cost.

PloyKoat Thermoplastic Polysteris a substituent to Bisphenol A BPA based resins and it’s currently been developed by Design Analysis Inc., Jacksonville, Florida in an effort to fully replace the current lining materials. PloyKoat is gaining the attention of many industries such as, the beverages industry, baby food, canned food and reusable water bottles. [17] The following points summarize the advantages of using PloyKoat thermoplastic polyester over BPA based lining:

- Uses 60% less energy in production than the conventional lining technique

- The coating doesn’t contain any harmful volatile organic compounds(VOC)s

- Requires only one layer of coating.

- FDA approved, and also has passed the Canadian regulations.

- High efficacy level in preserving beverages.

The general rule in improving the inner coating process, regardless of the material in use, is to apply only the adequate amount of coating, prevent the lining gases from escaping to the atmosphere and use the least energy in both the coating and recycling stages.

Reduce coating cost (use PloyKoat Thermoplastic Polyester)

- Eliminate scrapping process in recycling (use biodegradable materials)

Efficiency Improvement

- Reduce coating quantity of the interior coating

- Prevent epoxy resin from leaking to atmosphere

Health Improvement

Environmental Improvement

Figure 12: Summary diagram of the Improvement Analysis steps [18]

## 3.3.0 Aluminum Lid

For most metal food and drink containers, consumers pay for the metal itself which makes up 50-70% of the total container cost. Therefore, the amount of metal is the most significant cost item. The price of aluminum is high and the energy used in the manufacturing process is enormous. From the acquisition of virgin material to the creation of the can lid and its transportation, each step of the processing has an adverse effect on the environment.

### 3.3.1 Material Selection

The lid and body of the can are made separately. Whether it is a three-piece can or a two-piece can, how the lid is manufactured and attached to body remains the same. Aluminum beverage can ends are made from pre-coated coil and are stamped from coil in a press machine. Because of its flat geometry, the can’s end must be thicker and stiffer than the body to resist pressure. To strengthen the lid, can makers construct it from an alloy that has less manganese and more magnesium than that of the body. When magnesium and manganese are dissolved into the material, atoms displace some of the aluminum. Due to their slightly bigger size, the magnesium and manganese atoms distort the crystal latish. This distortion resists deformation which results in a stronger metal. Overall, the lid constitutes one fourth the total weight of the can. To reduce the mass, the diameter of the lid is cut down from 2.6 to 2.1 inches so that it is smaller than the diameter of the cylindrical body.

### 3.3.2Manufacturing Procedures

Aluminum Cans have either a plain end, requiring an opening tool or an easy-open end for convenience. Easy-open lids are mostly used for both beverage and food cans.  They are restricted to those cans where convenience is a marketing factor where the value of its content is slightly higher than one of an easy-open lid. The initial process for making plain and easy-open end are the same. Every can start off with a plain end or shell that will ultimately be converted into and easy-open end.  First of all, the plain ends are stamped directly from wide coil of metal or sheets cut from coil. The metal then goes through a process that produces multiple stamps for every stroke. Afterward, “the edges of the end shells are curled over to aid in the final operation of mechanical seaming the end onto the flange of the can”. [19]

When the curling is done, the end shells are passed through a line machine and a bead of liquid lining compound is applied in the curl to create a hermetic seal. The shells are then transferred to a conversion press where they will be made into easy-open ends.

The conversion step consists of scoring the perimeter of the opening panel, embossing the panels, forming the rivet and finally attaching the ring-pull tab. The pull tab is made from a narrow strip of pre-coated aluminum or steel in coil form by a multistage transfer press. It is pierced and cut when the riveting process detaches it from the strip and unites it to the end shell. However, the most critical operation is forming the rivet whose sole purpose is to secure the tab of the can. We start off by forming a bubble in the center of the lid. The flank of the bubble is then thinned to evenly distribute the metal. Afterward the bubble is redrawn to form a rivet. Finally the base of the lid is once again thinned to extend more metal into the button and the tab is placed over the upstanding rivet.

The finished cans are packed into paper sleeves and ready to be inspected before they are filled and sent to the market. Each unit of can delivered are identified to allow traceability to the production lot. If a defect is detected after packaging, it can easily be recalled. Packers make sure the quality of the containers meet the requirements.

Table 5: Summary of quality assurance checks on in-coming containers at food packers. [19]

### 3.3.3 Environmental Impact

The aluminum used in the manufacture of easy-open ends is developed to give the required mechanical properties. However, this alloy is subject to environmental stress cracking corrosion of aluminum due to its reaction with moisture which is accelerated by the presence of contaminants such as residual salts. The area around the pull tab is more susceptible to this form of cracking due to the stress it was subjected to. This adverse effect on the environment can be avoided if proper inspection, drying and filling of the can is done. Even in storage, humidity conditions need to be controlled by adequate ventilation systems. “Since rusting requires the presence of metal, oxygen and moisture” [19], external corrosion can be prevented by removing any of those factors and can be increased by the any of the following:

| Condensation due to temperature fluctuations, humidity | High chloride or sulphate | Changes, draughts, poor stacking |
| --- | --- | --- |
| Labels and paperboard divider sheets | Moisture from humid environment | Incomplete drying free water |
| Hygroscopic deposits | Alkali Detinning | Oxygen and Water |
| Low external tin coating and/or lacquer coverage | Too acidic or alkaline | Physical damage |
| Label adhesive | Damage to lacquer and tin coating of metal | High metal exposure |
| Rusty retorts and rust particles | Poor venting of retorts | Boiler water carry-over into retort |

Table 6 - Factors accelerating external corrosion [19]

External corrosion occurs at the weakest point on the can and any leaking neighbouring cans can affect uncontaminated cans and cause mechanical damages.

### 3.3.4 Improvements and Alterations

“Aluminum food cans are at present used where the customer appreciates the convenience of an easy-open container, or where a very long storage life is required such as for military or emergency stores. The first factor (convenience) could lead to some future growth in the demand for aluminum food cans.” [20] However, “manufacturers also sought to reduce the amount of aluminum needed” [21] by producing much smaller, thinner sheets. Because the metal represents half the cost of the can, recyclability has been a dominant factor on ways to use aluminum more efficiently.

## 3.4.0 External Can Design

Figure 13: Printing Automation [22]

Once the can has been stamped, coated and cleaned out, the next step is the painting process. The Aluminum Cans travel along an automated machine, where several ink fountains each supply different colour ink to different rollers which coat printer plates.  These printer plates are brought into contact with a rubber blanket and the design is translated to the blanket. [22] The Aluminum Cans are on a big wheel which spins the can at high velocity and are then brought into contact with the rubberized blanket. The can spins around as the label is applied, and a varnish coating is applied that makes the outside of the can shiny and protects the newly applied paint. [23] It is now moved onto the next process. The painting process is lightning quick averaging 1800 cans per minute.

The next step in the process is drying the ink. The cans are brought along a conveyor to a UV oven. The ovens are approximately 9 feet in length and operate at roughly 110F. The contain microwave energized mercury lamp which produce the Ultraviolet rays. The whole of the can, including the interior is exposed to the rays and dried in 0.7 seconds. [22]

### 3.4.1 Conventional Methods

Conventional printing methods incorporate the use of solvent based coatings or power coatings. These inks contain very high concentration of solvents, roughly 70-75% and a composed of hydrocarbons, ethers and acetates which, together produce Volatile Organic Compounds, VOC, and Hazardous Air Pollutants, HAPs. [22] Both compounds are considered harmful and place not only human lives at risk, but also cause numerous negative effects to the environment and ecosystem. This is primary reason these methods have been discontinued. Curing these kinds of inks require very high temperatures due to the high level of solids and solvent.

### 3.4.2 Economic and Energy Comparisons

Comparing conventional printing methods to present process, Today’s technology has vastly diminished its impact on the environment. Recent tests have shown the levels of VOC to be approximately 0.3 tons per billion cans. There is also a large reduction in CO2 emissions due to the lack of high heat ovens running on fossil fuels and creates no HAP by products.[22] Overall, UV inks have drastically increased the environmental friendliness of printing aluminum cans.

Analysing the economic side of the different process shows a significant increase in savings both in terms of savings and energy use when comparing thermal ovens and UV rays. The following figure describes the cost savings and energy savings.

Table 7: Economic Comparison. [22]

Table 8: Energy Use. [22]

## 3.5.0 Transportation

During the life cycle of the aluminum can, the ore and refined aluminum are transported multiple times. First, the Aluminum is extracted from ores such as bauxite in many countries around the world, including Australia, Brazil, china, and India. After the aluminum ore has been extracted from the earth it is then transported to a facility via truck or boat where the ore can be processed into pure aluminum, these facilities are normally within the country of extraction but this is not always so. After this process it is then transported to another facility designed to manufacture the parts needed to build aluminum cans which are aluminum sheets, these facilities are normally within the country where the can will be filled by a product such as a soft drink, and for our use could be transported from Australia, Brazil, china, and India all the way to North America. After the production of aluminum sheets comes the assembly of aluminum cans this process also takes place at a different facility then therefore increasing the pollution caused by transportation. Within the area of consumer use the final step of production occurs which is the filling of cans with a product such as soda. As shown, the different processes are all completed in different geographical areas. This greatly increases the environmental impact due to transportation.

### 3.5.1 Reduction of Transportation Emission

The transportation of aluminum and aluminums ore used to make aluminum cans cannot be done without the emission of harmful chemicals into the environment. Vehicles such as transport trucks and container ships are known to contribute significant amounts of air pollution to the environment through the emission of carbon monoxide and carbon dioxide. These toxins are known contributors to both the decrease of air quality (smog) and global warming. Additionally carbon monoxide is poisonous in high concentrations, preventing oxygen from binding to haemoglobin’s, which, in turn, reduces the oxygen carrying capacity of the blood leading to hypoxia. Although the effects of carbon monoxide poisoning seem extreme negative effects are only experienced in large concentrations. During the life cycle analysis of the aluminums can the vehicles can travel anywhere from 10,000kms to 15,000kms all the way from Australia for material extraction on to China for material processing then off to North America for the final steps of part manufacturing, assembly and product use. The primary sources of transportation for the different steps in the life cycle analysis are transported by trucks and container ships.  These modes of transportation require abundant amounts of energy and resources in the form of fuel to be used over the life cycle of aluminum cans. The emissions that result from these vehicles are destructive to the environment; therefore, one must take actions to reduce the overall impact that the transportation of aluminums cans has on the environment. Development within the area of engine efficiency will minimize the overall emissions from both the transport trucks and ships container used for transportation. There are other efficient ways to decrease emissions caused by transportation which is having all the processes take place relatively close to each other, this would include extraction, refinement of ore, part manufacturing, and finally assembly. After all, these processes are completed and the finished product can then be shipped off for the consumers various countries.

# 4.0.0 END LIFE OF ALUMINUM CAN

## 4.1.0 Decomposition Time

Aluminum cans can take 200-300 years to decompose and undergo chemical reactions that affect the environment. Therefore, this shows the advantage of using them and their environmental effectiveness. An aluminum can is worth about one penny, and about 50 percent of all the aluminum cans that are processed are recycled. [23]About two thirds of aluminum ever made is still in existence since it is one of the materials that do not lose its consistency. Recycling aluminum cans can save about 95 percent of the energy required to make a can out of virgin materials.

## 4.2.0 Comparison between stages

Impacts caused by packaging materials all depends on the material used to manufacture the container. Drinking containers underwent several improvements starting with a glass bottle to plastic one and leading to aluminum cans. It might seem that the differences between these containers do not matter. However, the main reason why this development occurred is to make the container more environments friendly and more efficient.

To begin with, manufacturing a glass bottle is a highly mechanized process, and the bottles in use today are no longer hand blown as they were in the past. A glass bottle is 100% recyclable and less energy is required to recycle a glass bottle than to create it again from raw materials, which as a result helps the environment. The reason why a glass bottle break when subjected to a shock is the hydrodynamic stress it undergoes.[23] However, glass bottle manufacturing takes place over several stages; the process starts from extracting raw material, melting, forming, annealing, to physical inspecting, quality control, machining and laser inspecting, to finally packing.

Figure 14: Glass Bottles [23]

### 4.2.1 Glass bottles

Second, plastic bottles are usually made of polyethylene; the intensity of polyethylene varies due to the properties of the bottle manufactured. When compared to other material like glass or metal, plastic polymers require higher processing. However, plastic is easier to use, requires less energy to extract from raw material, and is more environments friendly. Before recycling, most plastics are sorted according to their resin type. Some are also separated by color before they are recycled. The plastic recyclables are then shredded. These shredded fragments then undergo processes to eliminate impurities like paper labels. [24] This material is melted and often extruded into the form of pellets which are then used to manufacture other products.

Figure 15: Plastic Bottles [23]

### 4.2.2 Plastic Bottles

Therefore, this shows the advantage of using plastic bottles instead of glass bottles; since their recycling process and disposal process is easier than that of glass bottles. However, metal manufacturing is requires less energy than plastic manufacturing as mentioned above. Aluminum cans do not contain biphenyl A (BPA), found in plastic bottles. [32] This chemical has come under inspection due to a possible link to health risks that might include cancer. This leads to the conclusion that aluminum cans are by far the most favourable packaging items.

An aluminum can is a packaging item made primarily of aluminum. Most aluminum cans are made of two pieces; the bottom and body are drawn and ironed from a flat plate or shallow cup. After filling, the can end is sealed onto the top of the can. [31] The advantages of using aluminum cans are their light weight and high ranked aluminum properties. One of the properties of aluminum favoured in manufacturing is that it does not rust, which extends the life span of a can and decrease its impacts on the environment. [25] Aluminum cans are often made with recycled aluminum; approximately 68% of a standard can is recycled aluminum. Cans are the most recycled beverage container, at a rate of 69% worldwide. This explains why processing aluminum cans is more efficient than any other processes. However, one issue is that the top of the can is made from a blend of aluminum and magnesium to increase its strength, which makes recycling by melting almost impossible because the mixture obtained is unsuitable for recycling.  But, a new approach uses annealing to produce an alloy that works for both. [26]

Figure 16: Aluminum Cans [23]

### 4.2.3 Aluminum Cans

#### 4.2.3.1 Effects on Human Health and Ecosystems

Aluminum has not been demonstrated to pose a health risk to healthy humans. [26] However, it might affect patients, premature infants and elder population. Nevertheless, manufacturing process of aluminum directly affects human health. Some of these effects are:

Aluminum:

- Increases brain lesions in Alzheimer’s disease.

- Poses a hazard to patients with chronic renal failure and premature infants.

- Affects the hematopoietic system, the nervous system and bones.

- Produces microcytic, hypochromic anemia without iron deficiency.

- Creates rashes and arthritic pain if located in high quantities in drinking water.

- Causes nervous disorder.

- Causes dementia, pulmonary fibrosis, listlessness, etc.

- Causes lung and heart diseases.

Aluminum and its manufacturing process can also affect the ecosystem. Some of these effects are:

Aluminum:

- Releases toxic concentrations and contaminations into sedimentary cycle.

- Releases carbon dioxide which disturbs the environment.

- Might cause decrease in species diversity.

- Manufacturing requires tremendous energy which produces greenhouse gases, aggravating the “global warming”.[27]

- Transportation requires fuel consumption which causes air and water pollution.

- Leaches nutrients into soil which makes it acidic.

- Affects some plants that are main source of food for herbivorous species.

- Disturbs the environmental eco-chain which leads to disasters.

- Affects aquatic sources by releasing acidic substances.

- Affects fish and other aquatic creatures in several ways.

- Interferes with the ability of gills to take oxygen from water.

- Affects plant growth which leads to deforestation. [28]

#### 4.2.3.2 Waste Products and Energy Use

Approximately 31% of all aluminum produced comes from recycled scrap. To recycle aluminum, the cost of collection, [31] separation and recycling should be taken into account. Nonetheless, it produces significant cost savings compared to the production of new aluminum. The process for new aluminum includes mining, international shipping and manufacturing of raw aluminum. Energy used to recycle aluminum is approximated to be 5 % of the energy required to create it from bauxite. The amount of energy required to convert aluminum oxide into aluminum can be vividly seen when the process is reversed during combustion of thermite or ammonium perchlorate composite propellant. [29]Moreover, recycled aluminum could be said to create 5 % of the carbon dioxide produced in the creation from raw materials and the production of aluminum is attracted to sources of cheap electricity which causes hazardous effects on the environment. [30]

#### 4.2.3.3 Recycling Process

Cans are first divided from municipal waste, usually through an eddy current separator, and cut into little, equal pieces to lessen the volume and make it easier for the machines that separate them. Pieces are cleaned chemically/mechanically, and blocked to minimize oxidation losses when melted. Blocks are loaded into the furnace and heated to 750 °C ± 100 °C to produce molten aluminum. Dross is removed and the dissolved hydrogen is degassed. This is typically done with chlorine and nitrogen gas. [30] The furnace is tapped, the molten aluminum poured out, and the process is repeated again for the next batch. Depending on the end product it may be cast into ingots, billets, or rods, formed into large slabs for rolling, atomized into powder, sent to an extruder, or transported in its molten state to manufacturing facilities for further processing.

Figure 17: Pressed Cans [23]

## 4.3.0 Disposal

Recycling aluminum is the process by which scrap aluminum is reused after its initial production. It is simply done by re-melting the metal and using it to manufacture new products. The process uses much less energy needed to make new aluminum and also help to reduce pollution.  Aluminum recycling is now very common since its main use is for the manufacturing of cans (drinks/foods) and millions of them are disposed of daily. The use of recycled aluminum is also very economical in environmental terms as aluminum can be indefinitely recycled since the process does not transmute the material. Recycling aluminum cans in the United States have increased drastically every year shown in the figure below.

Table 9: U.S. Aluminum Beverage Can Recycling [32]

In a worldwide effort to decrease pollution, many cities across the globe have recycling programs especially for aluminum cans to improve its efficacy. Nowadays it is very common to have specific trash bins for collection cans only which helps to separate them from other waste and help in its recycling. However many are still dumped along with daily garbage in which case it is then separated at the municipal waste usually with the use of an eddy current separator . Money is also offered for return of several products such as metal caps, cans in an effort to urge the general population in the collection of the materials that can be recycled and is hence beneficial to everyone.

Once the cans are collected, they are either shredded into small pieces or crushed to increase storage capacity since a huge number is disposed of daily. The energy factor and transportation also have to be taken into consideration. While the cost and energy required recycling aluminum is low, in certain circumstances such as travelling long distances or the burning of fossil fuels to provide the energy required, the environmental impact can be devastative.

### 4.3.1 Waste

Aluminum thought it is not harmful to the environment on its own. It does not decompose but usually react with its surrounding such as oxygen to form aluminum oxide. The latter is not very harmful as well in its current quantities but both pure aluminum and its derivatives takes a long time to break down (normally around 75 years or more) due to heat and friction. It amasses in our landfills and in most cases it becomes nearly impossible to separate them from the other waste. Aluminum however does not impose a threat to the ecosystem it is part of unless it is designed to do so.

### 4.3.2 Public Recycling Program

Many cities and urban sectors now use recycling programs which attempts to collect certain materials and reuse them. This entire process starts with people deposing their used can into these containers, and periodically, a local recycling truck passes and collects everything from the recycling containers.  These trucks were specifically made for collecting recycling bins and also burn a lot of fuel for making local passes for collection and the final drive to the city’s collection station.  In these stations, the aluminum is separated and organized through a variety of systems.  Since aluminum is not magnetic, it cannot use an eddy current, and when mixed with trash, uses a lot of energy just to extract the aluminum. All of the recycled materials are organized into categories to make the final process as efficient as possible; this is done by loading everything into a hopper [35] and put through a conveyer belt.  All of this is either powered by electricity, or fossil fuels, with efficiencies ranging about 40-70%.  Workers then sort all of the materials into their own containers, which all these people need their own food to function, and are sometimes working long hours.  Specific materials such as aluminum cans are put through their own feed once categorized, and are eventually compressed into their own bail but another machine. [35] Once enough bails are collected, periodically these bails are also sent off, usually by another truck, to a treatment plant for aluminum.  To get an idea of the energies involved, there are separate companies that need to manage all of this chaos, and organize systems of trucks to perform certain tasks for this operation.  All of this requires fuel and for many recycled materials actually use more energy than the recycling can save.

Recycling aluminum is encouraged by allowing people to return cans to get refunds. However, the machines that accept cans for refunds are electrically powered and they tend to get dirty. Thus, it requires more maintenance than the average vending machine which also adds to the energy cost of this system.  On its own, it sorts all of the materials as soon as the consumer inserts the bottle or can and in this respect is very efficient compared to other recycling methods.  The machine also compresses and makes the bail when it is full of aluminum, and a worker simply removes it and when enough smaller bails are gathered, is ready to be picked up by a company that sends a truck to send all the aluminum bails to the aluminum treatment plant.

There are also community drop off centers that work similarly to local recycling, and encourage local citizens to drop off their aluminum cans and bottles and they eventually send them off to treatment plants as well.  There are also mobile versions of these programs that ask participating citizens to put their aluminum on the curb of their streets, and they pick them up.  All of these processes relies heavily on transportation, and in many ways are less efficient for individuals to transport their aluminum compared to larger trucks doing larger amounts of shipments in smaller trips.  Most of these alternatives end up in treatment plants as well.

### 4.3.3 Treatment Plants

Figure 18: Treatment Process [2]

The first part of the process is to clean the metals pieces to remove impurities and reduce oxidization. It is mostly done mechanically or by the use of chemicals at specific treatment plants. The collected aluminum is considered as scrap metal and is first shredded into small parts using a 1000hp shredder. [36] It is then passes through a powerful magnet to remove any metal fragments. A final step known as de-coating is required to remove the labels (paint/lacquer) usually made on products done by blowing hot air ( around 500 degrees)  on the shredded bits moving on a conveyer belt. The resulting gases are also recycled by afterburning it to the rest of the shredded aluminum to maintain efficiency and avoid relying fully on the machine that heats the air. [36] Once done, they are crushed into blocks as shown previously in Figure 17.

Table 10: Energy used to prepare aluminum scrap per 1000kg [2]

Table 11: Emissions, from processes, into the air per 1000kg [2]

The second part is the melting process where the blocks are loaded into a furnace heated at around 750 degree Celsius. This process melts the metal into a molten liquid and the heat further removes any impurities (dross – solid impurities floating in the liquid). The dross is then removed and the molten liquid can now be used for the production of new products. Is it usually made into ingots first then sent to different manufacturers for further processing into items such as cans, airplane parts and so on. [36] Once all the aluminum is liquefied, the impurities float to the top, and are removed, and the whole furnace tilts to be poured into a cast to form these aluminum ingots, taking 3 hours to cool, are 18-ton each [36] and are 25 feet long. [37]

As mentioned before, aluminum can be recycled indefinitely. Hence the new products can once again be recycled in the same way as above over and over again.  Recycling aluminum is hence absolutely efficient compared to other metals. In the following years, as our technology evolves, the recycling processes will be refined to further reduce the negative effects and increase its efficiency.

Table 12: Energy used to cast 1000kg of aluminum rolling ingot [2]

Table 13: Top 8 pollutants emitted to the air per 1000kg of aluminum rolling ingot [2]

Table 14: Top 8 pollutants emitted into water per 1000kg of aluminum rolling ingot [2]

Recycling aluminum is absolutely efficient, especially compared with extracting, processing, and using raw aluminum and other metals.  Aluminum is also very versatile because of its unique properties.  Processing aluminum from recycled aluminum “saves 95 percent of the energy used to make aluminum cans” [38] compared to processing it from raw material. The most inefficient part of the recycling process is the fuel burned from transportation.  Transportation consists mainly of gathering, organizing, and bringing the recycled aluminum to its appropriate destination. [39] As time progresses, new technologies will develop which will make recycling more efficient and environmentally friendly than the present recycling system. Aluminum is one of the best and most efficient materials to recycle because it can be reproduced 100% of the initial scrap.

# 5.0.0 CLOSING REMARKS

The life cycle of aluminum cans is very interesting because it can have an impact on the environment and be environmental friendly when recycled. It takes a lot of energy and resources to mine aluminum and process it. However, when recycled, only a mere 5% of energy is needed to make aluminum cans compared to processing raw material. Today, almost 2/3rd of all aluminum cans are recycled with the help of environmental advertisements and the simplicity of the recycling process. Aluminum is unique metal in the sense that it possesses many qualities and properties that makes it more environmentally friendly than other metals. Aluminum cans are easy to recycle, very light in weight which reduces transportation costs and have a relatively low melting point to allow for less energy casting into ingots. There are still many areas of the Life Cycle Process that can be improved on, especially when it comes to transportation. Compared with alternative materials for beverages such as glass and plastic, aluminum is the most ideal because it takes least amount of raw materials, less energy to create, can easily be recycled and outputs fewer pollutants that contribute to global pollution. Processing from raw material is very harmful in terms of emissions and energy usage. At our present time, we must use the technology we currently possess to innovate and fix the current problems in the aluminum life cycle process to ensure a safe and healthy future.

Figure 19: Aluminum Can Lifecycle Process [41]

# 6.0.0 ENVIRONMENTAL BENEFITS

Aluminum cans creates many positive impacts on the environment in comparison with tin or steel cans. For example, recycling aluminum will save energy, it reduces carbon footprint, and recycling aluminum satisfies the increasing demand for aluminum.

Firstly, According to the International Aluminum Institute, aluminum is recyclable. From the total amount of aluminum that is created, about seventy-five percent of this is still in use today. As a result, aluminum cans are the most valuable item in the municipal waste.

Secondly, According to Steve Larkin, president of the Aluminum Association, recycling cans uses less energy.  It uses 95% less energy to recreate the aluminum can and it also emits 95% less greenhouse gases.

Finally, nowadays the producer uses less aluminum to create the tin cans. Steel and tin cans are presently similar to the technology of aluminum cans. Today, the aluminum can weighs 0.0466oz which is less than what they used to use in 1993. The manufacturer has decreased it by 15%, which helps them save a lot of aluminum, which helps them produce more aluminum cans. As a result, there are able to produce more for the population. As the population increasing, the demand for materialism is high also. This approach that they made on this aluminum tins will satisfy the needs of the striving population.

To conclude, aluminum cans are really easy to recycle. Therefore, aluminum cans will bring forth a positive impact to our environment.  It saves a lot of material and the manufacturers are able to recover most of the aluminum by recycling. As a result, they are able to save a lot of energy, and it will make our planet earth sustain its environmental life condition.

# REFERENCES

  - Economy and Ecology of Aluminum. London: the University of Wilson sin Press.

  - “Life Cycle Impact Assessment of Aluminum Beverage Cans.” PE Americas (2010): Final Report. N.p., n.d. Web. 7 June 2013.
<http://www.container-recycling.org/assets/pdfs/aluminum/LCA 2010-AluminumAssoc.pdf>.

  - The Material Forum and the Metals Society (1982). Aluminum and Its Future Patterns of use in Great Britain. London: The Chameleon Press Ltd.

  - “Providing the alumina industry an eco-friendly and efficient red mud technology.” Weir Minerals. N.p., n.d. Web. 7 June 2013. <http://www.weirminerals.com/PDF/brochure%20geho%20alumina%20industry.pdf>.

  - “Aluminum Limited (1953).” Aluminum Panorama. Montreal: Aluminum Limited

  - “Arsenic removal from water/wastewater using adsorbents--A critical review.” Mohan, D., Pittman, CU. & Hazard Mater, J. (Apr 2007) 142 (1–2): 1–53. N.p., n.d. Web. 5 June 2013. <http://www.ncbi.nlm.nih.gov/pubmed/17324507>.

  - "Waste materials used in concrete manufacturing: Red Mud Utilization.” Chandra, Satish. 
pp. 292–295 (1996-12-31) ISBN 978-0-8155-1393-3.

  - "Red Mud Disposal: Handbook of Aluminum recycling.” Schmitz, Christoph. pp. 18. (2006) ISBN978-3-8027-2936-2.

  - “Key Improvements to Hall·Heroult since the End of World War II”. Kai Grjotheim et al. JOM (November 1995), Volume 47, Issue 11, pp 32-35 ISSN: 1047-4838 (Print).

  - "V-Rod." PennState, The Pennsylvania State University (2010), Pennsylvania, United States.     N.p., n.d. Web. 8 June 2013. <http://www.personal.psu.edu/lat5088/edsgn100/cans.html>.

  - “Rolling aluminum: from the mine through the mill.” Aluminum association, Inc. (2007). N.p., n.d. Web. 7 June 2013. <http://www.aluminum.org/Content/NavigationMenu/TheIndustry/SheetPlate/Rolling_Aluminum_From_The_Mine_Through_The_Mill.pdf>.

  - U. S. Environmental Protection Agency, Metal Can Manufacturing--Surface Coating, September 1998, 52.

  - “The History of Can Making: Overview.” Can Manufacturer Institute. N.p., n.d. Web. 7 June 2013. <http://www.cancentral.com/hist_overview.cfm>

  - 2010, Robin MackarFebruary. "Environmental Factor: February 2010: HHS Calls for More Research on BPA." National Institute of Environmental Health Sciences (NIEHS). N.p., n.d. Web. 9 June 2013. <http://www.niehs.nih.gov/news/newsletter/2010/february/spotlight-hhs.cfm>.

  - Guzman, Doris de. "Development of BPA alternatives is increasing." Chemical Industry News & Chemical Market Intelligence. N.p., n.d. Web. 9 June 2013. <http://www.icis.com/Articles/2010/03/24/9345229/development+of+bpa+alternatives+is+increasing.html>.

  - "Bisphenol A-free polymer coating touted as alternative to epoxy can lining." Food Processing news (food process, food equipment, food machinery). N.p., n.d. Web. 9 June 2013. <http://www.foodproductiondaily.com/Packaging/Bisphenol-A-free-polymer-coating-touted-as-alternative-to-epoxy-can-lining>.

  - "AFFIRM Act Cuts Wasteful Insurance Subsidies, Increases Transparency | Environmental Working Group." Environmental Working Group. N.p., n.d. Web. 9 June 2013. <http://www.ewg.org/release/affirm-act-cuts-wasteful-insurance-subsidies-increases-transparency>.

  - Elawad, Ali. Aluminum Can, Improvement analysis, 2012

  - Bev. Page, Mike Edwards and Nick May, Metal Cans, 2001, 130-150.

  - Eric Wootton, Alcan Deutschland GmbH, Göttingen, Case Study on Can Making, 1994. Pg 30.

  - William F. Hosford and John L. Duncan, The Aluminum Beverage Can, 1994. Pg 52.

  - Brady, Bob. "UV-Curable Coatings for Aluminum Can Production." Division of Pollution Prevention and Environmental Assistance. N.p., n.d. Web. 8 June 2013. <http://www.p2pays.org/ref/01/00843.pdf>.

  - "How Aluminum Cans Are Made." Alcoa RPD: About Alcoa Rigid Packaging: How Aluminum Cans Are Made. Alcoa RPD. N.p., n.d. Web. 3 June 2013. <http://www.alcoa.com/rigid_packaging/en/about/making_cans.asp>.

  - Cornwall and Isles of Scilly Health Authority. Clayton, Barbara, and Chairman Cornwall. Rep. HMSO London: n.p, 1991. Print.

  - "Aluminum--Is it Eco-friendly or a Bio-hazard..?." Find Science & Technology Articles, Education Lesson Plans, Tech Tips, Computer Hardware & Software Reviews, News and More at Bright Hub. N.p, n.d. Web. 9 June 2013. <http://www.brighthub.com/environment/science-environmental/articles/43121.aspx>.

  - Agency for Toxic Substances and Disease Registry, United States. Toxicological Profile for Aluminum. N.p, n.d. Web. 9 June 2013. <http://www.atsdr.cdc.gov/toxprofiles/tp22.pdf>.

  - New Hanover Country. "Home." Home. N.p, n.d. Web. 9 June 2013.  <http://www.nhcgov.com/>.

  - "Report on Water Pollution at Lowermoor." Advising Group. Lowermoor Incident Health Advisory Group. 1989.

  - Rubin, Edward S., and Cliff I. Dacidson. Introduction to Engineering and the Environment. Vol. 1st Edition. Boston: Mc Graw Hill, 2001. Print.

  - Stobart PD. Centenary of the Hall & Héroult Processes. Rep. St.Albans: International Primary Aluminium Institute, 1886 - 1986. Print.

  - "Types of Cans | Can | Technical Information | TOYO SEIKAN." Toyo Seikan Kaisha, Ltd.. N.p, n.d. Web. 9 June 2013. <http://www.toyo-seikan.co.jp/e/technique/can/kind/>.

  - “U.S. Aluminum Beverage Can Recycling” aluminum.org The Aluminum Association. N.p, n.d. Web. 9 June 2013. <http://www.aluminum.org/Content/NavigationMenu/NewsStatistics/StatisticsReports/UsedBeverageCanRecyclingRate/UBCRecyclingRate.pdf>.

  - “Highest recycling rate and recycled content of any beverage container.” The Aluminum Association, N.p,  n.d. Web. 9 June 2013. <http://www.aluminum.org/AM/Template.cfm?Section=Home&TEMPLATE=/CM/ContentDisllay.cfm&CONTENTID=30442>.

  - Ward NI. Environmental Contamination of Aluminium and Other Elements in North Cornwall as a Result of the Lowermoor Water Treatment Works Incident. Rep. Vol. . . Vol 1: 118-121. Geneva: International Conference, 1988. Print.

  - “Recycling: How Does it Work?” recycle.georgetown.org Georgetown Texas. N.p.,  n.d. Web. 9 June 2013. <http://recycle.georgetown.org/recycling-how-does-it-work/>.

  - “Guidance for the Identification and Control of Safety and Health Hazards in Metal Scrap Recycling.” Occupational Safety and Health Administration. N.p.,  n.d. Web. 9 June 2013. <https://www.osha.gov/Publications/OSHA3348-metal-scrap-recycling.pdf>.

  - “How is Aluminum Can Recycled?” earth911.com, Earth911, N.p.,  n.d. Web. 9 June 2013. <http://earth911.com/recycling/metal/aluminum-can/how-is-an-aluminum-can-recycled/>.

  - “Recycling Fun Facts.” Can Manufacturers Institute, N.p., n.d. Web. 7 June 2013. <http://www.cancentral.com/funfacts.cfm>.

  - “Aluminum Can Recycling.” WasteCare Corporation: Trash Compactors, Balers, Recycling Equipment, Waste Equipment and Related Products. N.p., n.d. Web. 7 June 2013. <http://www.wastecare.com/Articles/Aluminum_Cans_Recycling.htm>.

  - Ducray, Eric. Manufacturing of Aluminum Can. Aluminum Sheet Flow Chart, 2012.

  - Younes, Souha. Aluminum Can. Aluminum Can Lifecycle Process, 2012.